IT6016 Network Security Practical Tasks

Network Plan

Nicholas Rowley

09/04/2019

Table of Contents

IPv4 Addressing Scheme	1

Device Network Configuration	1

Network Topology Diagram	2

Device Authentication and other Details	3

Routers, Switches and ASA (General login / ASDM details) (LOCAL only)	3

ACS	3

PCS	3

AAA Logins (TACACS Server on ACSBuzzNick) (For CLI Access and ASDM Access)	3

Clientless SSL VPN	3

SNMP	3

NTP	3

Access Lists (R3)	4

Access Lists (Switches)	4

OSPF Authentication	4

ASA Password Encryption	4

ASA Access Lists	4

ASA Interface Names	4

Site to Site VPN Encryption	4

Screenshots	5

Test 1 - PC’s in subnets can ping servers in other subnets	5

PC1	5

PC3	6

Test 2 - Ping’s from router 2 (r2) and router 1 (r1) cannot reach devices in subnets	7

R1	7

R2	8

Test 3 - Pings from PC1 cannot reach PC2 (in the DMZ zone)	9

Test 4 - Syslog and SNMP data from Cisco devices is being recorded on the Syslog/SNMP server	10

Syslog	10

SNMP	11

Test 5 - Connections to manage routers using requires SSH and telnet does not work	12

Test 6 - Logging onto a switch and router r3 successfully uses accounts from the ACS TACACS+ server - Buzzadmin1 account	12

Router AAA	12

Switch AAA	13

Test 7 - PC2 in the DMZ zone cannot access the Windows 10 PC	13

Test 8 - Traffic from site 1 to site 2 successfully travels across the VPN tunnel and there are VPN sessions	14

Test 9 - Windows 10 PC can successfully connect in a secure browser session to the web portal on the ASA using the BuzzVPNuser account	15

Test 10 - Router r2 can make a SSH connection to manage the ASA	16

GNS3 window with network topology	17

ASDM running with a connection to the ASA	18

Using the ACS	19

Network Devices and AAA Clients page (under Network Resources)	19

Users page (under Users and Identity Stores, Internal Identity Stores)	20

Authorization page (under Access Polices, Access Services, Default Device Admin)	21

# IPv4 Addressing Scheme

| Subnet Name | Network ID with CIDR |
| --- | --- |
| R1 to R2 | 130.10.1.200 /30 |
| R2 to R3 | 130.10.1.204 /30 |
| Site 1 Outside (VLAN 2) | 204.150.175.0 /29 |
| Site 1 Inside (VLAN 3) | 192.168.1.64 /27 |
| Site 1 Management VLAN (S1BuzzNick) | 192.168.1.0 /27 |
| Site 1 Management VLAN (S2BuzzNick) | 192.168.1.32 /27 |
| Site 1 VLAN 4 | 192.168.1.96 /27 |
| Site 1 VLAN 5 | 192.168.1.128 /27 |
| Site 2 Management VLAN (S3BuzzNick) | 192.168.2.0 /27 |
| Site 2 VLAN 2 | 192.168.2.32 /27 |

Site to Site VPN uses Interfaces: ASA IP – 204.150.175.2 & R3 IP – 130.10.1.206

Site to Site VPN uses local IP subnet addresses from Site 1 and Site 2.

# Device Network Configuration

| Device Name | Interface | VLAN | IPv4 Addresses |
| --- | --- | --- | --- |
| R1 | Gig 0/1 Gig 0/0.2 Loopback 0 | N/A 2 | 130.10.1.201 204.150.175.1 192.168.0.10 |
| R2 | Gig 0/0 Gig 0/1 Loopback 0 | N/A N/A | 130.10.1.202 130.10.1.205 192.168.0.20 |
| R3 | Gig 0/0.2 Gig 0/0.93 Gig 0/1 Loopback 0 | 2 93 N/A | 192.168.2.33 192.168.2.1 130.10.1.206 192.168.0.30 |
| ASABuzzNick | Gig 0/0.2 Management 0/0.91 Gig 0/1.3 Gig 0/1.92 Management 0/0.4 Management 0/0.5 | 2 91 3 92 4 5 | 204.150.175.2 192.168.1.1 192.168.1.65 192.168.1.33 192.168.1.97 192.168.1.129 |
| S1BuzzNick | VLAN 91 | 91 | 192.168.1.2 |
| S2BuzzNick | VLAN 92 | 92 | 192.168.1.34 |
| S3BuzzNick | VLAN 93 | 93 | 192.168.2.2 |
| ACSBuzzNick | E0/0 | 4 | 192.168.1.98 |
| PC1BuzzNick | E0/0 | 5 | DHCP |
| PC2BuzzNick | E0/0 | 3 | DHCP (Preferred – 192.168.1.66) (Public IP – 204.150.175.3) |
| PC3BuzzNick | E0/0 | 2 | DHCP (Preferred – 192.168.2.34) |

# Network Topology Diagram

# Device Authentication and other Details

## Routers, Switches and ASA (General login / ASDM details) (LOCAL only)

Enable Password: BuzzAccess6016!

Local Login: admin

Local Password: BuzzCisco123

## ACS

Local Login (CLI): admin

Password (CLI): BuzzAccess6016!

Web GUI Login: acsadmin

Web GUI Password: Password2

## PCS

Password: Passw0rd!

## AAA Logins (TACACS Server on ACSBuzzNick) (For CLI Access and ASDM Access)

| Login | Password | Enable Password | Group |
| --- | --- | --- | --- |
| BuzzAdmin1 | BuzzCisco123 | BuzzAccess6016! | Admin |
| BuzzAdmin2 | BuzzCisco123 | BuzzAccess6016! | Admin |

AAA List name (Routers and Switches) – AAALIST

TACACS Server Name (Routers / Switches / ASA) – BuzzTACACS

TACACS key (ACS[ALL DEVICES]) – BuzzTACACS6016

## Clientless SSL VPN

Profile Name: BuzzSSLVPN

Policy Name: Buzz-SSL-VPN-POLICY

Bookmark: Web-Server (https://192.168.1.66)

Username: BuzzVPNuser

Password: BuzzSys123

## SNMP

Version: 3

Username: SNMP-Admin

Password: BuzzSNMP6016!

Privacy Password: BuzzSNMPEncrypt6016!

SNMP group: SNMP-Buzz-Group

Host: 192.168.2.34

View Name: SNMP-RO

## NTP

Trusted-Key: 6016

Password: BuzzNTP

## Access Lists (R3)

ALLOWVTYACCESS – Access list for input traffic on vty lines.

VPN-TRAFFIC – Traffic that is allowed from SITE 2 across the site to site VPN

ALLOW-OUTSIDE-VPN-TRAFFIC – Traffic sent from outside of site 2 that is allowed into the LAN

SNMP-ACCESS – Subnets that are allowed to view SNMP for this device

## Access Lists (Switches)

ALLOWVTYACCESS – Access list for input traffic on vty lines.

SNMP-ACCESS – Subnets that are allowed to view SNMP for this device.

## OSPF Authentication

Key – 1

Password – BuzzRouting6016

## ASA Password Encryption

Encryption Password – BuzzEncrypt6016

## ASA Access Lists

OUTSIDE-IN-TRAFFIC – Traffic from the outside interface that is allowed on the LAN.

## ASA Interface Names

| ASA Interface Type & Number | ASA Interface Name | ASA Security Level | Subnet |
| --- | --- | --- | --- |
| Management 0/0.4 | INSIDE-VLAN-4 | 100 | 192.168.1.96/27 |
| Management 0/0.5 | INSIDE-VLAN-5 | 100 | 192.168.1.128/27 |
| Management 0/0.91 | Inside-Management-1 | 100 | 192.168.1.0/27 |
| Gig 0/0.2 | OUTSIDE | 0 | 204.150.175.0 /29 |
| Gig 0/1.3 | DMZ | 70 | 192.168.1.64/27 |
| Gig 0/1.92 | DMZ-Management-2 | 100 | 192.168.1.32/27 |

- Allow interfaces with the same security levels to send traffic freely between each other.

## Site to Site VPN Encryption

- Transform set: esp-aes esp-sha-hmac

- Hash: sha

- Group: 2

- Lifetime (seconds): 86400

- Integrity: sha

- Authentication: Pre-share

- Encryption: aes 256

- Isakmp key: BuzzVPNKEY6016

- Interfaces: 204.150.175.2 (ASA) to 130.10.1.206 (R3)

# Network Device Running Configurations

Stored in the ‘IT6016_91030261_Running_Configs’ folder.

# Screenshots

## Test 1 - PC’s in subnets can ping servers in other subnets

### PC1

### PC3

## Test 2 - Ping’s from router 2 (r2) and router 1 (r1) cannot reach devices in subnets

### R1

### R2

## Test 3 - Pings from PC1 cannot reach PC2 (in the DMZ zone)

## Test 4 - Syslog and SNMP data from Cisco devices is being recorded on the Syslog/SNMP server

### Syslog

### SNMP

## Test 5 - Connections to manage routers using requires SSH and telnet does not work

## Test 6 - Logging onto a switch and router r3 successfully uses accounts from the ACS TACACS+ server - Buzzadmin1 account

### Router AAA

### Switch AAA

## Test 7 - PC2 in the DMZ zone cannot access the Windows 10 PC

## Test 8 - Traffic from site 1 to site 2 successfully travels across the VPN tunnel and there are VPN sessions

## Test 9 - Windows 10 PC can successfully connect in a secure browser session to the web portal on the ASA using the BuzzVPNuser account

## Test 10 - Router r2 can make a SSH connection to manage the ASA

## GNS3 window with network topology

## ASDM running with a connection to the ASA

## Using the ACS

### Network Devices and AAA Clients page (under Network Resources)

### Users page (under Users and Identity Stores, Internal Identity Stores)

### Authorization page (under Access Polices, Access Services, Default Device Admin)